# AI-Based Fire Detection & Monitoring System

Raspberry Pi | Sensors | AI | GSM | Web Dashboard

## 1. Project Overview

This project implements a real-time fire detection system using multiple sensors, rule-based logic, and an AI (Logistic Regression) model.
It monitors temperature, smoke, and flame, triggers local alarms, sends SMS alerts, and displays live data on a web dashboard.

The system is designed to reduce false alarms by combining:

- Sensor thresholds (rules)

- AI risk prediction

## 2. System Features

Real-time temperature monitoring (DS18B20)
 Smoke detection using MQ-2 via MCP3008 ADC
 Flame detection (digital sensor)
 AI-based fire risk prediction
 Buzzer alarm on detection
 SMS alert using GSM module
 Web dashboard (Flask)
 Multi-threaded for high reliability

## 3. Hardware Components

| Component | Description |
| --- | --- |
| Raspberry Pi | Main controller |
| DS18B20 | Temperature sensor |
| MQ-2 | Smoke/Gas sensor |
| MCP3008 | Analog-to-Digital Converter |
| Flame Sensor | Digital flame detection |
| GSM Module (SIM800/900) | SMS alerts |
| Buzzer | Audible alarm |

## 4. Software & Libraries

### Python Libraries

RPi.GPIO

busio

board

adafruit-mcp3xxx

numpy

joblib

flask

serial

### OS Requirements

- Raspberry Pi OS

- 1-Wire enabled (for DS18B20)

- SPI enabled (for MCP3008)

## 5. System Architecture

Sensors ──► Threads ──► Shared Data ──►

├─ Rule Engine ──► Alert

├─ AI Model ─────► Risk Score

├─ GSM ─────────► SMS

└─ Flask ───────► Web Dashboard

## 6. Configuration Parameters

FLAME_PIN = 17

BUZZER_PIN = 27

TEMP_THRESHOLD = 50.0

SMOKE_FACTOR = 1.5

AI_THRESHOLD = 0.70

MODEL_PATH = "lr_model.joblib"

PHONE_NUMBER = "+201062948826"

### AI Normalization (must match training)

MAX_TEMP = 100.0

MAX_MQ2 = 60000.0

## 7. Sensor Details

### 🔥 Flame Sensor

- Digital input (GPIO 17)

- Detects direct flame presence

### 🌡 DS18B20 Temperature Sensor

- Reads from Linux 1-Wire filesystem

- Output in °C

### 💨 MQ-2 Smoke Sensor

- Analog output

- Read via MCP3008 (SPI)

- Calibrated at startup to define baseline smoke level

## 8. MQ-2 Calibration Process

At startup:

- Sensor warms up for 15 seconds

- 50 readings are averaged

- Baseline is calculated

- Smoke threshold is defined as:

SMOKE_THRESHOLD = BASELINE × SMOKE_FACTOR

This adapts the system to the environment.

## 9. AI Model Logic

### Model Type

- Logistic Regression (loaded from lr_model.joblib)

### Input Features

| Feature | Description |
| --- | --- |
| t_n | Normalized temperature |
| s_n | Normalized smoke |
| f | Flame (0 or 1) |
| ratio | Smoke / baseline |
| dt_n | Change in temperature |
| ds_n | Change in smoke |

### Output

Risk ∈ [0.0 – 1.0]

## 10. Alert Decision Logic

### Rule-Based Alert

Triggered if any condition is true:

- Flame detected

- Smoke exceeds threshold

- Temperature exceeds threshold

### AI-Based Alert

Triggered if:

AI risk ≥ 0.70

### Final Alert

Alert = Rule Alert OR AI Alert

## 11. Alarm & SMS System

### 🔔 Buzzer

- Turns ON during alert

- Turns OFF when safe

### 📩 SMS Alert

- Sent via GSM module

- Cooldown: 30 seconds

- Message includes:

  - Temperature

  - Smoke level

  - AI risk score

Example:

🔥 FIRE ALERT!

Temp: 62.4C

Smoke: 38420

Risk: 0.91

## 12. Multi-Threading Design

| Thread | Purpose |
| --- | --- |
| flame_thread | Reads flame sensor |
| mq2_thread | Reads smoke |
| temp_thread | Reads temperature |
| ai_thread | Calculates AI risk |
| alert_thread | Controls buzzer & SMS |
| web_thread | Runs Flask server |

Shared data is protected using a threading lock.

## 13. Web Dashboard

### URL

http://<RaspberryPi-IP>:5000

### Features

- Live temperature

- Smoke level

- Flame status

- AI risk probability

- Safe / Danger status

### API Endpoint

GET /api

Returns JSON:

{

"temperature": 45.2,

"mq2_value": 23140,

"flame": false,

"risk": 0.32,

"alert": false

}

## 14. Program Execution Flow

- Initialize GPIO & SPI

- Calibrate MQ-2 sensor

- Load AI model

- Start all threads

- Monitor continuously

- Trigger alarms & alerts

- Serve web dashboard

## 15. Shutdown Handling

- Press CTRL + C

- GPIO is safely cleaned up

- Program exits gracefully

## 16. Future Improvements

- Cloud logging

- Mobile app

- Camera integration

- Retrain AI with real fire data

- Push notifications

## 17. Conclusion

This system combines hardware sensors, AI intelligence, and real-time web monitoring to provide a robust fire detection solution with reduced false alarms and rapid response.